TrakBox ver 2.00		May.05 '92

TrakBox ver 1.50d is released as official TrakBox code with the ver

number 2.00.

TrakBox ver 1.50d		Apr.30 '92

1.Bug in calculation through Dec.31 to Jan.01 next year fixed.

TrakBox ver 1.50c              Apr.19 '92

1.Bug in kepele uploading which has more than 60 satellite data.

Fixed.				      Thanks bug report =>JR1ING

2.Minor change in satellite list display.

5 satellite display in one line changed to 4 satellite each line.

3.Minor change was made for more reliable update in station information

uploading.(need line number at the first colum)

TrakBox ver 1.50b              Apr.18  '92

1. Station information update by file uploading supported.

In menu-6-9, after system reset, TrakBox prompt you for station info

file uploading. Ctrl-Z returns preveous menu without update.

Station information file format is as follow.

Wish from many STNs

; 1-st line (station elements)

;   CALL_SIGN,LATITUDE,LONGITUDE,HEIGHT,rotor_zero_position

; 2-nd line (rotor ADC calibration)

;   Azimuth_mini,Az_max,Elevation_mini,El_max

; 3-rd line (deadband and skew value)

;   Az_dead_band,El_dead_band,Az_skew,El_skew

; 4-th line (CAT port configuration)

;   CAT_rig,Full/AOS_control,ICOM_rx_address,ICOM_tx_address

; Each token must be delimited with tab or space character.

; Each line must be started from top colum.

;      For TrakBox     JA6FTL   Apr.14.1992

;

JA6FTL  27.391  128.657  50  South

8  859  1  442

3  1  0  0

ICOM  AOS  1E  3F

2.You can configure CAT or mic-click doppler correction for each satellite.

In menu3-1, select CAT or MIC option. CAT is default.

With this enhansment, one additional token is needed in freq/mode data

file.                                        Wish from KD8SI,KC6YPM

AO-13 435.500 145.900 usb lsb cat

AO-16 437.025 145.940 usb fm mic

3.Bug fixed in manual rotor control (menu-6-6).

To down the elevation rotator, value 0 was not accepted in the old version.

Thanks bug report => KE0VZ

4.Minor enhansment in satellite selection list.

In menu3-2 the selected satellite is marked with '*'.

Wish from KC6YPM

TrakBox ver 1.50		Apr.12  '92

1. Frequency and mode data update by file uploading supported.

In menu-3,after uploading kepele data file, TrakBox prompt for

freq/mode data file uploading. The file format is as follow.

Wish from many STNs

AO-13 435.500 145.900 usb lsb

AO-16 437.025 145.940 usb fm

DO-17 145.825 0 fm none

WO-18 435.075 0 usb none

LO-19 437.150 145.840 usb fm

FO-20 435.910 145.850 usb fm

AO-21 145.983 435.016 usb fm

UO-22 435.120 145.900 fm fm

Satellite name is case sensitive.

Each line has 5 tokens delimeted with space character.

Sat_name,dwnlink_freq,uplink_freq,dwnlink_mode,uplink_mode.

Each line must be started at 1-st colum.

2. Full orbit or AOS CAT controle selection.

In the Radio port configuration menu, this option is selectable.

In the case of AOS CAT control, the CAT command is issued in AOS state.

TrakBox ver 1.40a		Mar.29  '92

1. Minor change in schedule display.

Scroll option can be selected.

2. Bug in Rotor lock at 180 deg. fixed

Sometimes, the rotator locks at 180 degrees position. fixed.

3. Minor change in satellite selection with ADC.

CN9-pin6 must be grounded in the case of ADC.

TrakBox ver 1.40		Mar.28  '92

1. Doppler correction for uplink frequency was supported.

F2 at menu 3-1 is the base frequency for calculation.

Every 500Hz shift of calculated frequency produce one CAT command.

2. YAESU CAT command enhansment.

YAESU CAT need adequate COM port transfer speed. YAESU manual request

50-200mSec interval in every CAT byte. But in real test, 10mSec is

acceptable. In TrakBox we chose 12mSec.

TrakBox ver 1.30e              Mar.22  '92

1. Bug in YAESU CAT command in v1.30d was fixed.

2. Added optional selection for YAESU CAT.

In the case of modified wider pass band FM filter for FSK(uo-22),

and replaced with original FM narrow band filter, downlink mode

must be set to FMN position. In menu 6-7, you can select this option.

CAT Rig KENWOOD,YAESU,ICOM (K/Y/I) ?  Y

Select FMN mode ? [Y/N] _

-------------------------------------------------------------------------

Any comment,bug report or proposal for TrakBox will be welcome.

de JA6FTL @ uo-22    CI$: 71701,251