TrakBox ver 2.20 beta          Jan.15 '92

1.New routine for rotator over heat protection.

In rotator control, if rotator SW on state continue 5 min. without ADC

read value change,  TrakBox issue warning message and beep and off the SW

instantly. This watch dog function is to protect the rotator motor from

burn when the antenna movement is disturbed by some external reasons.

The argolithm is very simple, but effect is very large in the case of

rotator emmergency.

System checks the rotor SW at every 00 sec. with the IRQ signal from RTC

If ON detected, flag is set. This checking continues 5 times (5min).

After 5 times check done, if every check detect ON then System read the ADC

value and compare it with the initial ADC value. If the difference of these

two value does not exceed dead band value, System decides as there happened

some trouble in rotator movement. The cource is displayed on your terminal

lower right corner as "E0-5".

Warning message with beep tone is issued on terminal and LCD display.

The tracking mark replaced with "E" and this state is not recovered without

system reset.

5 minites may be heared too long, but too short interval causes false

positive reaction in slow satellite such as AO-13.

You can check the function in real time tracking menu or direct rotor

control menu by switch off the rotator controler.

2.minor bug fixed

AOS indicator LED flickers in parking.  => fixed.

AOS indicator mal function in multi tracking. => fixed.

TrakBox ver 2.12 beta	        Dec.25 '92

Minor bug were fixed.

1.Bug in North start.  => EA4PE

2.Minor change of LCD display. Az=>AZ, El=>EL.

TrakBox ver 2.11 beta          Dec. 03 '92

1.Automatic step pulse generation at AOS for mic click RX.

At AOS, TrakBox calculate the step number and send step pulse for doppler

correction.  With this function, Mic click RX can set the initial AOS

frequency for automation.  This is done with following sequence.

F1: doppler base freq.      F_AOS: calculated freq. at AOS

F_LOS: calculated freq. at preveous orbit LOS

step_size:Freq. step

After satellite selection, F1 is set in memory and when the object satellite

AOS,  (F_AOS-F1)/step_size = pulse number.  A series of up pulse with this

number are genarated.  At LOS of this satellite, F_LOS is memoried untill

next orbit AOS.   In next AOS, (F_AOS-F_LOS)/step_size is the pulse number.

This sequence assumes that the doppler correction is done perfectly in the

full AOS span.  It will be done by T_B ADC or other analog AFC circuit.

Must set [Y] in menu6-8 and MIC click On in menu3-1, and Tracking on (*).

If proper step pulse are generated, you can see this in your terminal.

fdx/f_diff/pulse:435.120000/9772/97

UP pulse:*_*_*_*_*_*_*_*_*_*_*_*_*_*_*_*_*_*............

(In this case, F1=435.120, step size = 100Hz then 97 pulse generated)

2. New AOS indicator port.

Now, P16 (CN3-7) reflects the AOS status.   When the elevation value reach

to Blink angle, this terminal is become 0(GND) level.

In using this option, you need adequate interface to control the PC/TRX

power line.  Using U6 spare gate for buffer is good idea.

3. Preset antenna control

Main menu 8-3, you can preset the antenna direction.

4. Other minor changes.

LCD display format changed. [Az 197.7 EL-12.9]  => 4X1AS

User selectable step control polarity. Normal/Reverse by menu6-8.

GND active(for KENWOOD) is Normal and Floating active is Reverse(for YAESU,

need external pull up R).

TrakBox ver 2.09  beta         Nov. 22 '92

1.Offset for F1/F2 introduced. This function is mainly for down converter.

If offset value is configured, Freq.CAT data is corrected with this value.

Example:

;

AO-13S 2400.730/-2256.000 435.620 usb lsb mic ma

;

In AO-13 mode-S, Using down converter with local oscillator 2256.000MHz,

the issued freq. set data is 144.730MHz(2400.730-2256.000).

/(slush)[+-][Offset_freq] is needed.

Default is 0.000. Configurable from menu3-1 and FREQ.DAT file.

2.Phase value display on LCD.

Now, LCD display format is automaticaly selected.

If defined as "ma" in FREQ.DAT file, the LCD display format is

[Axxx Eyy Pzzz.z]  A:Azimuth E:Elevation P:Phase

TrakBox ver 2.08B beta         Nov. 18 '92

1.Direct rotor control from terminal.

In main menu-8, you can control the rotor control SW by key.

"4" : Left

"6" : Right

"8" : Up

"2" : Down

"5" : Stop

any other key : return to main menu.

The antenna direction value is displayed on both terminal and LCD.

TrakBox ver 2.08 beta          Nov. 13 '92

1.RS232C port.

Now, 1200/2400/4800/9600 bps supported. You can select the baud rate

in main menu 4 (Station Data). New rate take effect after power on reset.

Default is 9600bps. If CN9-pin8 is grounded, the rate is fixed to 1200bps.

* When first install of new ROM, sometimes the COM port rate is not initialized

properly. In this case, remove the battery backup (JP13) and clear the

RAM data temporarily and power on again.

2.In realtime tracking mode, "?" key-in from terminal returns the object

satellite name.

3.The antenna control starts when the EL value reaches to a certain degree.

You can set this value from STATION.DAT file.  Default is -2.0.

; 5-th line (Antenna parking, limit inf and rotor control start EL value.)

;   5,1/0(antenna park ON/OFF),park-Az,park-El,1/0(limit ON/OFF),Az-min,Az-max,

;     El-min,El-max, ANT control start EL value

5 1  270  0   1   270  90   0   45  -2.0

; (with this configulation, antenna parking position is Az270, El0

;  Antenna movement is limited within 270(West) - 0(North) - 90(East)

;  and elevation 0 - 45 degrees. If you do not need limit, set the SW 0

;  Antenna control start when EL value reaches -2.0 degrees.

TrakBox ver 2.07 beta          Nov. 04 '92

1.Reported bugs were fixed

RTC hopping problem at xx:59:59 fixed ( => JA1OGZ)

2.Now, no need jumper-10.

3.Now, Standalone mode and Terminal mode were separated completely.

No key in from terminal is accepted in standalone mode.

4.[Phase] value is available in LCD display.

In main menu 4, you can set P(hase) option at Rotor start set.

SP or NP set the option.  ( Mainly for AO-13 user => JH1AOY)

TrakBox ver 2.06D       	Oct. 28 '92

1 Minor change in main menu command.

M(ulti track) and S(ingle track) command from main menu.

P(riority track) toggle command in multi track mode.

Added direct antenna control command from main menu.

2.Reported bugs were fixed.

Rotator North start problem fixed ( => EA4PE)

NASA format file with header text problem fixed ( =>DL1CR,ZL1UNM)

Station data file updating problem fixed ( => DL1CR)

IC-970 freq. control problem was fixed   ( => KA7CMF)

3 v2.05 and 8031/8051

v2.05 could not work properly with 8031/8051 due to embedded RAM size

problem => fixed.

4.Tracking mark

* : single track mode

! : multi track without priority

& : multi track with priority

5. [NOTICE!]

you must set jumper10(located near X-tal) 1-2.  RTC send interrupt signal

by every 60sec vir this jumper for priority tracking calculation.

TrakBox ver 2.05		Oct. 18 '92

1.Added priority tracking function in multi track mode.

When priority flag is set in menu3-4, in the case of multiple satellites

are AOS, the TrakBox check the each satellite position every 1 minites and

change the object automatically according to the priority order.

This function achieved by interrupt from RTC every 60sec.

[NOTICE!]

you must set jumper10(located near X-tal) 1-2.

*beta version anounce the priority check every one minute. This is  a

temporary expedient for beta tester to make sure the function.

2.Improvement for YAESU CAT problem.

YAESU CAT system has bug.  FT-736 can not set the same frequency for uplink

and downlink band. In  the  case of  frequency change from AO-13 to UO-22,

it is needed to escape the RX frequency to another frequency band.

TrakBox solved this problem by setting the RX freq. to dummy 1296MHz band

in initial mode/freq. setting sequence.     ( Thanks advice => DL1CR,JA1OGZ)

TrakBox ver 2.03c              Oct. 05 '92

1.Host mode supported.

Main menu-7 lead you TrakBox host mode.

By sending ascii simple command from com port, TrakBox set the antenna direct

-ion(AZ/EL) and issue rig control command(UP/DWN mode,UP/DWN frequency and

user defined character) to TRX with it's word. Az/El value and command words

are displayed on LCD.

[Supported commands]

Azxxx Elxxx : Azimuth and Elevation (ex. Az220 El 45)

MUxxx       : Uplink mode          (MULSB,MUFM)

MDxxx       : Down link mode       (MDUSB,MDFM)

FUxxxxxxxx  : Uplink freq.         (FU14595000 10Hz order)

FDxxxxxxxx  : Downlink freq.       (FD43590000 10Hz order)

Sxxx        : any character.       (STX;,SFN0;)

Q           : return to main menu

?           : help for command words.

TrakBox ver 2.02j		Sep. 15 '92

[Additional functions]

1.Multiple satellite tracking supported.

By menu driven configuration, max 5 satellites can be calculated at the

same time and if some satellite AOS, the control shift to single satellite

tracking mode. After LOS, the control return to multiple tracking mode.

This function can be set from terminal by entering 'M'(ultiple) in single

track mode and 'S'(ingle) in multiple track mode returns to single track mode.

In standalone mode, just change the satellite selection SW to 0(zero)

position toggles the  mode.

To indicate the working mode, the tracking status mark is "*" in single track

mode and "!" in multiple track mode.

The satellite for multiple track must be selected as #1-#5(max) in selection

menu 3-2 and the number in menu 3-4.

2.Automatic assignment of satellite selection SW.

By uploading satellite info file(FREQ.DAT), the satellites are assigned

as selected satellite.

3.Antenna parking and Antenna movement limit.

Antenna parking position and movement limit can be set from menu 6-4.

4.minor change was done in STATION.DAT file format.

*each line need line number in the line head.

*additional line (#5) added for antenna parking and antenna movement

limitation.

; 5-th line (Antenna parking and limit info.)

;   5,1/0(antenna park ON/OFF),park-Az,park-El,1/0(limit ON/OFF),Az-min,Az-max,

;     El-min,El-max

; Each token must be delimited with tab or space character.

;

;      For TrakBox     JA6FTL   Oct.05.1992

;

1 JA6FTL  27.391  128.657  50  South

2 8  859  1  442

3 3  1  0  0

4 KENWOOD  AOS  1E  3F

5 1  270  0   1   270  90   0   45

5.RS-232C port 1200 baud supported.

At start up, grounding port-A bit7(CN9 pin8) enables 1200baud.

This is for remote control via land line MODEM.

[minor debug]

1.BS key full suported in deleting miss typed character.

TrakBox ver 2.01a              June 25 '92

Bug in uploading the station element file was fixed.

TrakBox ver 2.00(dated May.10) May.10 '92

Minor tuning was done in RTC initialization. Now,no clock delay in

system reset.

TrakBox ver 2.00(dated May.02) May.05 '92

Minor tuning was done in mic_click operation. Initial CAT command now

issued at  -2.00> EL >-3.00

TrakBox ver 2.00(dated Apr.30) May.05 '92

TrakBox ver 1.50d is released as official TrakBox code with the ver

number 2.00.

TrakBox ver 1.50d		Apr.30 '92

1.Bug in calculation through Dec.31 to Jan.01 next year was fixed.

TrakBox ver 1.50c              Apr.19 '92

1.Bug in uploading kepele file which has more than 60 satellite data was fixed.

Fixed.				      Thanks bug report =>JR1ING

2.Minor change in satellite list display.

5 satellites display in one line changed to 4 satellites each line.

3.Minor change was made for more reliable update in station information

uploading.(need line number at the first column)

TrakBox ver 1.50b              Apr.18  '92

1. Station information updating by file uploading supported.

In menu-6-9, after system reset, TrakBox prompt you for station info

file uploading. Ctrl-Z returns previous menu without update.

Station information file format is as follow.

Wish from many STNs

; 1-st line (station elements)

;   CALL_SIGN,LATITUDE,LONGITUDE,HEIGHT,rotor_zero_position

; 2-nd line (rotor ADC calibration)

;   Azimuth_mini,Az_max,Elevation_mini,El_max

; 3-rd line (deadband and skew value)

;   Az_dead_band,El_dead_band,Az_skew,El_skew

; 4-th line (CAT port configuration)

;   CAT_rig,Full/AOS_control,ICOM_rx_address,ICOM_tx_address

; Each token must be delimited with tab or space character.

; Each line must be started from top column.

;      For TrakBox     JA6FTL   Apr.14.1992

;

JA6FTL  27.391  128.657  50  South

8  859  1  442

3  1  0  0

ICOM  AOS  1E  3F

2.You can configure CAT or mic-click Doppler correction for each satellite.

In menu3-1, select CAT or MIC option. CAT is default.

With this enhancement, one additional token is needed in freq/mode data

file.                                        Wish from KD8SI,KC6YPM

AO-13 435.500 145.900 usb lsb cat

AO-16 437.025 145.940 usb fm mic

3.Bug fixed in manual rotor control (menu-6-6).

To down the elevation rotator, value 0 was not accepted in the old version.

Thanks bug report => KE0VZ

4.Minor enhancement in satellite selection list.

In menu3-2 the selected satellite is marked with '*'.

Wish from KC6YPM

TrakBox ver 1.50		Apr.12  '92

1. Frequency and mode data update by file uploading supported.

In menu-3,after uploading kepele data file, TrakBox prompt for

freq/mode data file uploading. The file format is as follow.

Wish from many STNs

AO-13 435.500 145.900 usb lsb

AO-16 437.025 145.940 usb fm

DO-17 145.825 0 fm none

WO-18 435.075 0 usb none

LO-19 437.150 145.840 usb fm

FO-20 435.910 145.850 usb fm

AO-21 145.983 435.016 usb fm

UO-22 435.120 145.900 fm fm

Satellite name is case sensitive.

Each line has 5 tokens delimited with space character.

Sat_name,dwnlink_freq,uplink_freq,dwnlink_mode,uplink_mode.

Each line must be started at 1-st column.

2. Full orbit or AOS CAT control selection.

In the Radio port configuration menu, this option is selectable.

In AOS CAT control, the CAT command is issued in AOS state.

TrakBox ver 1.40a		Mar.29  '92

1. Minor change in schedule display.

Scroll option can be selected.

2. Bug in Rotor lock at 180 deg. fixed

Sometimes, the rotator locks at 180 degrees position. fixed.

3. Minor change in satellite selection with ADC.

CN9-pin6 must be grounded in the case of ADC.

TrakBox ver 1.40		Mar.28  '92

1. Doppler correction for uplink frequency was supported.

F2 at menu 3-1 is the base frequency for calculation.

Every 500Hz shift of calculated frequency produce one CAT command.

2. YAESU CAT command enhancement.

YAESU CAT need adequate COM port transfer speed. YAESU manual request

50-200mSec interval in every CAT byte. But in real test, 10mSec is

acceptable. In TrakBox we chose 12mSec.

TrakBox ver 1.30e              Mar.22  '92

1. Bug in YAESU CAT command in v1.30d was fixed.

2. Added optional selection for YAESU CAT.

In modified wider pass band FM filter for FSK(uo-22),

and replaced with original FM narrow band filter, downlink mode

must be set to FMN position. In menu 6-7, you can select this option.

CAT Rig KENWOOD,YAESU,ICOM (K/Y/I) ?  Y

Select FMN mode ? [Y/N] _

-------------------------------------------------------------------------

Any comment,bug report or proposal for TrakBox will be welcome.

de JA6FTL @ uo-22    CI$: 71701,251