TB-CNTRL  Written by Chris Jackson ZL2TPO

This program will run with SatSked (and other automatic satellite

selection software) and will switch the TrakBox rotors on and off

before and after a pass.

The program uses the TrakBox terminal mode and does not make use

of the multi tracking mode. The command to enable the rotators

is placed in the satellite batch file and hence will only run

when SatSked runs a pass. This ensures that TrakBox will always

be tracking the same satellite as SatSked has run, and if a pass

is edited out of SatSked's pass file, TrakBox will not track it,

thus reducing wear on the rotors.

To run TB-CNTRL the following perameters are required

TB-CNTRL <port> <speed> <int> <satellite>

<port> is the HEX address of the com port to which TrakBox is

connected.

COM1 = 3F8

COM2 = 2F8

COM3 = 3E8

COM4 = 2E8

<speed> is the baud rate TrakBox is using.

<int> is the interrupt request line to use.

IBM defaults are -: COM1 4

COM2 3

COM3 4

COM4 3

<satellite> is the satellite name or number. This can take

the form of the satellite name as seen in TrakBox's

database (eg. UO-22), and is not case sensitive.

It can also be the number of the satellite as you

would see it when entering the realtime tracking mode,

preceded with a # (eg #1, for satellite 1).

To stop the rotators from tracking enter the satellite

name STOP.

TB-CNTRL will place TrakBox into Realtime tracking mode

and track the desired satellite. It then checks to see if

the rotators are enabled, and if not, enables them.

The program perameters allow the use of non standard com

port addresses and interrupt request lines.

I have tested the program on a 33MHz 386DX and a 25MHz 386SX

without any problems. I ran TB-CNTRL under DesqView on the

SX with no problems.

I would appreciate hearing of any bugs or problems that users

may find. This program has undergone limited testing and

shouldn't present any problems.

-------------------------------------------------------------

Changes

-------

Interrupts have been enabled in the program. I use the same

interrupt for PB/PG and TrakBox. TrakBox is connected the COM3

and PB/PB uses COM1. After TB-CNTRL has commanded TrakBox it

returns the interrupt line to normal so it can be used by

other programs. As I never control TrakBox at the same time

that I use PB/PG, this works fine. It sames me a valuable IRQ

line.

I found that some times PG and PB didn't reset the com port

properly when they exit. If TB-CNTRL was using the same

interrupt it was unable to receive characters. To reset the

com port that PB/PG I wrote a program called CLR-PORT.

This disables the interrupts from the selected port and

clears an output on the 8250 ACE which seems to be the main

offender in tying up the com port. If you have problems

running TB-CNTRL after PB/PG, and you are using the same

interrupt request line on different ports then try using

CLR-PORT.

It perameters are -:

CLR-PORT <address> <int>

<address> is the HEX address to be reset eg.

the port PB/PG uses.

<int> is the interrupt number it uses.

NOTE: If you enter the wrong address or interrupt number

strange things could happen.

I have now installed a routine to check if the rotators are

(E)nabled.

The latest version also has DesqView freindly code installed.

During any waiting time, TB-CNTRL gives time back to other

programs running under DesqView. This has no effect if

DesqView is not running.

If anyone has any problems with using this new version, please

let me know.

73's Chris ZL2TPO @ KO-23

@ UO-22